- Course Introduction & Quartus/ModelSim Setup

- MAX 10 FPGA and DE10-Lite Board Overview

- SystemVerilog Modules & Continuous Assignments

- always_comb vs assign Coding Styles

- always_ff, always_latch & Sequential Logic Basics

- Blocking vs Non-blocking Assignments

- Case Statements & Priority Encoders

- Finite State Machines: Moore & Mealy Designs

- Testbench Development and Simulation

- Using built in templates.

- Parameterization & Generate Constructs

- Synchronous Counters and Timers

- Debouncing Keys and Switches

- LED and Seven-Segment Display Drivers

- PLL Generation with IP Catalog

- On-Chip RAM (Single/Dual-Port) Integration

- ROM Creation and MIF/HEX Initialization

- Simple UART Transmitter/Receiver

- PWM Audio Output with On-Board Buzzer

- VGA Signal Generation (Optional)

- Hierarchical Design & Top-Level Integration

- Synthesis, Fitting & Timing Analysis in Quartus

- Pin Assignment and .qsf Constraints

- In-System Memory Editor & SignalTap Logic Analyzer

- Project Packaging and Version Control

- Capstone Project: Mini-Game Console on DE10-Lite

# Course Introduction

Learning logic design is your passport to the silicon-powered world that underpins everything from smartphones to self-driving cars. In this course you start with the nuts-and-bolts of digital circuits, build those ideas on the re-programmable fabric of the Intel MAX 10 FPGA that lives on the DE10-Lite board, and finish by driving a VGA display with your own retro-style game. Along the way you will experience the rapid “edit–compile–flash–see it blink” cycle FPGAs offer, while also seeing where ASICs still rule for mass-production, ultra-low-power designs. By the last lesson you will be able to turn ideas into real hardware, speak SystemVerilog with confidence, and walk away with a portfolio project that lights up any résumé.

Why Logic Design Still Matters

Digital systems are built from deterministic building blocks—gates, flops, memories, and finite-state machines. Mastering those blocks lets you understand and influence every performance and power number that software alone cannot touch. FPGAs expose these primitives directly, letting you see timing, placement, and resource utilisation instead of guessing.

Why We Use an FPGA

- Immediate iteration – load a new bitstream in seconds; no wafer run or mask cost required.

- Low entry cost – a DE10-Lite kit is roughly $140 retail or $82 academic, versus millions for a first ASIC tape-out.

- Built-in lab gear – onboard USB-Blaster, SDRAM, ADC, LEDs, switches and direct VGA output mean you can prototype whole systems without extra hardware.

- Rich IP catalogue – Quartus ships free PLLs, RAMs, UARTs and more that drop into your design with a few clicks. (Intel)

FPGA ↔ ASIC: The Real Trade-offs

| Aspect | FPGA | ASIC |
| --- | --- | --- |
| Flexibility | Reprogrammable after manufacture | Fixed at tape-out |
| Unit cost | High for volume | Lowest for >100 k units |
| Power / Speed | 2-10× slower & higher power due to routing fabric | Optimised transistors, highest perf/W |
| Time-to-Market | Days to weeks | 6-18 months |

Your Journey Through the Course

- Fundamentals – binary logic, combinational vs. sequential circuits, synchronous design rules.

- SystemVerilog Core – modules, assign, always_comb, always_ff, blocking vs. non-blocking, parameterisation.

- Design Patterns – debounced inputs, counters, UARTs, RAM/ROM, PLL clocking, VGA timing.

- Verification – self-checking test-benches in ModelSim, waveform‐driven debugging.

- Hardware Bring-up – Quartus fitter, timing analysis, SignalTap in-system probing.

- Capstone – build a complete retro game (think Pong or Space Invaders) that generates 640 × 480 VGA video from your FPGA, stores sprites in on-chip RAM, and reacts to real-time input.

What You Will Gain

- Hands-on fluency with SystemVerilog and the Intel Quartus tool-chain.

- A mental model for timing, metastability, and resource budgeting that transfers directly to ASIC flows.

- A portfolio artifact—a working hardware game—that demonstrates creativity, RTL discipline, and board-level debugging skills.

- A springboard into advanced topics such as multi-clock systems, DSP blocks, and hardware acceleration for AI.

By the end, “digital logic” will not be an abstract diagram on a whiteboard—it will be a colourful game bouncing across a monitor, driven by circuits you designed and silicon you programmed.

# Quartus/ModelSim Setup

Lesson 1 – Course Introduction & Quartus / ModelSim Setup (Quartus Prime Lite 17.1)

Goal: Equip every learner with a ready-to-use Intel® Quartus Prime Lite 17.1 + ModelSim-Intel® FPGA Edition tool-chain that can synthesize, program and simulate designs for the DE10-Lite (MAX 10 10M50).

1. Why stick with 17.1 Lite?

- 17.1 is the last “classic” release officially bundled with a free ModelSim-Intel FPGA Edition starter license and full MAX 10 support, making it ideal for introductory labs. (Intel)

- Later Quartus versions broke some old tutorials and slightly changed the GUI; staying on 17.1 keeps the screenshots and scripts in this course 100 % reproducible.

2. Prerequisites

| Item | Minimum Spec |
| --- | --- |
| OS | Windows 10/11 64-bit |
| Disk | ≈ 10 GB free (Quartus + ModelSim + device files) |
| RAM | 8 GB (16 GB recommended for faster compiles) |
| Board | Terasic DE10-Lite with on-board USB-Blaster II |

(Quartus 17.1 Lite itself is license-free; no floating license server needed).

3. Download packages

- Look for Quartus download page:
https://www.intel.com/content/www/us/en/software-kit/669444/intel-quartus-prime-lite-edition-design-software-version-17-1-for-windows.html

Optional - Create / sign-in to an Intel FPGA account (free).

- Browse to Intel’s download page and pick Quartus Prime Lite Edition v17.1 (Windows)

- Go to Individual files and download 3 files: 
- Intel® MAX® 10 Device Support
- ModelSim-Intel® FPGA Edition (includes Starter Edition)
- Intel® Quartus® Prime (includes Nios® II EDS)
These files should be in the same directory, i.e. downloads.

4. Install Quartus Lite 17.1

- Run QuartusLiteSetup

- Accept the EULA, keep default path C:\intelFPGA_lite\17.1.

- In Device Selection, tick MAX 10 only (saves ~3 GB).

- Finish and allow the installer to create desktop shortcuts.

5. if you downloaded Modalism, it will Install Modalism-Intel FPGA Edition automatically.

- When asked for a license, choose “Starter Edition (free)” – no file needed.

6. USB-Blaster II Driver (Windows only)

- Plug the DE10-Lite via micro-USB.

- When Windows asks for drivers, point to
C:\intelFPGA_lite\17.1\drivers\usb-blaster-ii.

- Verify in Device Manager that “Intel FPGA Download Cable II” appears.

7. First-run sanity check

- Open Quartus Prime Lite 17.1.

- Tools ▸ Programmer → Hardware Setup → select USB-Blaster II.

- Click Auto Detect – the 10M50DAF484C7G device should pop up (DE10-Lite FPGA).

8. Typical bug with the driver:

Having trouble with programming your FPGA? In some cases, the board might not be recognized under the Select Device section in the Programmer window. The main cause for that is drivers that weren't installed correctly (for the Kiwi and the DE10-Lite), due to Windows updates (most of the time).

To solve that issue, you'll need to manually install the drivers through the device manager.

To install the drivers:

- Plug your board into your PC.

- Open Device Manager.

- Make sure that the board shows up as USB-Blaster under Other Devices. If your board doesn't show up, check your cables and try another USB port on your PC.

- Right click on the USB-Blaster device, and select Update Driver.

- Select Browse my computer for drivers.

- Go to your Quartus Prime installation directory and select the Drivers folder. (IntelFPGA_Lite > 17.1 > Quartus > Drivers) Don't go deeper within the Drivers folder.

- After selecting the Drivers folder, click next until you're done*.

- Go back to the Programmer Window in Quartus prime, select Hardware Setup and select your board. If your board doesn't show up, make sure that your drivers were installed properly, check your cables and try other USB ports.

*If you get the "Windows encountered a problem installing the drivers for your device" error, you'll need to enable unsigned driver installation. You can follow this tutorial for installing unsigned drivers: https://www.maketecheasier.com/install-unsigned-drivers-windows10/

Lets try our first project to check that we have the system in place ready to learn and play.

# Hello world – simple switch led.

Learning how to open, compile, and run your first “Hello-World” FPGA design is the quickest way to feel the full edit--compile--flash cycle.
In this chapter you will (1) pull the ready-made DE10_lite_17_basic_gate_verilog.qar from GitHub, (2) restore it in Quartus Prime Lite 17.1, (3) compile and download the bit-stream to the DE10-Lite board, and (4) inspect the top-level file so you understand what each line does.
By the end, flipping SW[0]–SW[2] will light LEDR[0]–LEDR[2] (with LED 2 inverted), proving your tool-chain and board are alive.

1 Get the project from GitHub

- Navigate to the course repository and press Code ▸ Download ZIP or git clone https://github.com/avisalmon/FPGA_design_store.git.

- Un-zip / clone into any workspace folder; you will see the archive DE10_lite_17_basic_gate_verilog.qar.
A .qar is a Quartus Archive that packs every file needed to rebuild the project in one compressed bundle.

2 Open the .qar in Quartus 17.1

| Option | Steps |
| --- | --- |
| A (inside Quartus) | File ▸ Restore Archived Project… → select DE10_lite_17_basic_gate_verilog.qar. Quartus explodes the archive, creates a fresh project folder, and automatically opens it. (Intel) |
| B (double-click) | On Windows the .qar extension is already associated with Quartus; just double-click the file and the same restore wizard appears. |

Tip – keep projects portable: later you can snapshot your own work with Project ▸ Archive Project to generate a new .qar

3 Compile and create the bit-stream

- Open the restored project (DE10_lite_17_basic_gate) in Quartus.

- Confirm the target device is 10M50DAF484C7 (Processor ▸ Device).

- Press Start Compilation (or ⌘/Ctrl + L). Quartus runs analysis, synthesis, fitting, and timing; you should end with “Full compilation was successful” and a new .sof file in output_files/.

4 Download to the DE10-Lite board

- Plug the mini-USB cable into JTAG USB-Blaster II and power the board.

- Quartus Tools ▸ Programmer → Hardware Setup → select USB-Blaster II.

- Click Auto Detect – you should see 10M50DAF484 appear.

- Add File → choose DE10_lite_17_basic_gate_verilog.sof, tick Program/Configure, press Start.

- Flip SW0 / SW1 / SW2 on the board:

  - SW[0] → LEDR[0] (direct)

  - SW[1] → LEDR[1] (direct)

  - SW[2] → LEDR[2] (inverted)
Your first LED demo is live! (ftp.intel.com, YouTube)

5 Inside the top-level file

assign LEDR[0] = SW[0];          // pass-through

assign LEDR[1] = SW[1];          // pass-through

assign LEDR[2] = !SW[2];         // active-low inversion

5.1 Generated scaffold

The header banner and the long list of ENABLE_* macros come from Terasic System Builder. That utility writes a board-specific wrapper so you can enable/disable peripherals at compile time. (Terasic)

5.2 Port list

Every physical pin the board breaks out—clocks, SDRAM, HEX displays, keys, LEDs, VGA, Arduino header—appears in the module port list and is guarded by the same compile-time defines. Only the signals you need are compiled, keeping the fitter fast.

5.3 Continuous assignments

The three assign statements are continuous drivers: the FPGA fabric hard-wires each LED to the corresponding switch, updating every clock-cycle without software. That is the canonical “Hello world” of hardware.

6 Verify and experiment

- Try editing the file to light multiple LEDs (assign LEDR = SW;), re-compile, and re-program; you will see all switches mirrored.

- Explore RTL Viewer (Tools ▸ Netlist Viewers) to see the literal wires the compiler produced.

- Open Pin Planner to view physical pin names (PIN_F15, PIN_A14, …) that connect to each LED or switch.

7 What you gained

- Tool-chain confidence: Quartus, USB-Blaster, and the DE10-Lite board are confirmed working end-to-end.

- Project hygiene: you know how to restore and archive .qar projects, critical for sharing labs and handing in homework.

- First RTL insight: you have seen the skeleton of a Terasic top module, the use of parameterised defines, and the simplest possible use of the assign statement.

Keep this project folder; in later lessons you will extend it with an always-clocked counter, a finite-state machine, RAM blocks, a PLL, and eventually a full VGA video game.

# MAX 10 FPGA and DE10-Lite Board Overview

The Intel MAX 10 family marries non-volatile flash configuration, low-power fabric, on-chip analog, and rich clocking into a single chip; the Terasic DE10-Lite board drops the highest-density 10M50 device onto a student-friendly PCB packed with SDRAM, VGA, an accelerometer, Arduino & GPIO headers, and an integrated USB-Blaster II. Together they give you a self-contained playground that boots in milliseconds, edits in minutes, and scales from “hello-LED” to full-screen VGA games—perfect for every lab in this course. Intel CDRDDE10-LiteTerasicIntelIntel CDRD

MAX 10 FPGA at a Glance

Architecture & Resources

- Flash-based, single-chip FPGA—configuration stored on-die; instant-on without external memory.

- Up to 50 k Logic Elements (LEs), 1.6 Mbit M9K RAM, 144 × 18-bit DSP multipliers, and 4 independent PLLs in the 10M50 device used on DE10-Lite.

- Dual 12-bit ADCs for temperature sensing or external analog sampling—unique among low-cost FPGAs.

- Hardened flash supports dual-image storage (factory + application) for fail-safe updates.

Clocking & Timers

- Four PLLs provide frequency synthesis, phase shifting, and clock domain crossing.

- Internal 8 MHz RC oscillator for quick prototypes when no crystal is available.

DE10-Lite Board Hardware

Core Components

| Part | Details | Why it matters |
| --- | --- | --- |
| FPGA | MAX 10 10M50DAF484C7G (484-pin FBGA) | Largest MAX 10 variant → plenty of logic for VGA game |
| Memory | 64 MB 16-bit SDR SDRAM (ISSI IS42S16400) | Frame buffers, game assets, large state machines |
| Clock | 50 MHz crystal on CLK1 & CLK2 inputs | Feeds PLLs for VGA (25 MHz) or audio PWM |
| USB-Blaster II | On-board programming & JTAG debug | No external cable needed; SignalTap works out-of-box |
| I/O | 10 slide SW, 10 red LEDR, 2 KEY push-buttons, six 7-seg HEX | Ideal for combinational/seq-logic labs |
| Video | 4-bit R / G / B DAC, HS, VS → VGA connector | Drives 640 × 480 @ 60 Hz for retro game project |
| Sensors | ADXL345 3-axis accelerometer (SPI) | Adds interactive input without extra hardware |
| Expansion | 40-pin GPIO header & Arduino UNO-R3-compatible header | Plug in shields, PMODs, or breadboard jumpers for custom peripherals |

Power & Configuration

- Board accepts 5 V via barrel jack or USB mini-B; on-board regulators supply 3.3 V and 1.2 V rails. Intel FTP

- MAX 10’s flash lets the board boot in < 50 ms and supports remote field updates through JTAG or On-Chip Flash IP. Intel

Development Workflow Highlights

- Terasic System Builder generates the top-level wrapper with only the peripherals you enable, reducing compile time.

- Quartus Prime 17.1 recognises the USB-Blaster II automatically; pin assignments for every LED, switch, and VGA line are pre-mapped in the supplied QSF.

- SignalTap Logic Analyzer taps any internal net through the same JTAG cable—crucial for debugging FSMs and video timing. Terasic

Why This Platform for the Course

- All-in-one lab bench: no external clock generators, programmers, or displays required—ideal for classrooms and home study.

- Bridges theory to practice: students can move from simple LED gating to pipeline-heavy VGA graphics on the same silicon.

- Path to ASIC concepts: instant recompile cycles expose timing closure, power walls, and area trade-offs that mirror real ASIC flow—minus the masks.

Armed with these resources, you’re ready to explore SystemVerilog fundamentals, instantiate RAMs and PLLs from the IP Catalog, and ultimately drive a full retro game engine—proving your designs not just in simulation but in glowing pixels on a real monitor.

# Creatin a project

Option 1 – from scratch. New project

Option 2 – from existing par or

Option 3 – from other project settings.

# SystemVerilog Modules & Continuous Assignments

SystemVerilog divides hardware description into modules that talk to each other through typed ports, and it lets you describe pure-combinational logic inside those modules with the assign continuous-assignment statement. Modules give structure and hierarchy; assign yields simple, single-line logic that is evaluated “all the time” in hardware. After this chapter you will know when and how to use each—and you will test yourself by building a 4-bit adder/comparator that lights the DE10-Lite LEDs with only continuous assignments.

1 Modules – the building blocks

A module is the fundamental unit of design in Verilog/SystemVerilog; it encapsulates internal signals, parameters, and the ports that connect it to the outside world

module mux2 #(parameter WIDTH = 8)                // generic width

(input  logic [WIDTH-1:0] a, b,

input  logic             sel,

output logic [WIDTH-1:0] y);

assign y = sel ? b : a;              // continuous assignment

endmodule

Key ideas

- Parameter list #(...) lets one definition service i.e.  many widths and depths.

- Port list (...) defines each input/output: direction, data-type (logic or wire) and width.

- Modules can be instantiated inside other modules

2 Continuous assignments (assign)

assign drives a net (wire, or the 4-state logic when it has only one driver) with the value of an expression continuously meaning hardware is fabricated so that the output reflects the expression at all times.

assign parity = ^data_in;      // XOR-reduce for parity bit

assign led    = ~button_n;     // invert push-button for active-high LED

Rules of thumb

- The left-hand side must be a net type when multiple drivers could exist; use logic for single-driver convenience.

- No event list, delays, or blocking/non-blocking keywords—assign is strictly combinational and synthesizable as long as no timing controls are added.

- It is perfect for glue logic, bus multiplexers, priority encoders, decoders, simple arithmetic—anything that does not need memory.

3 assign vs always_comb

Both model combinational hardware, but they differ in style and capability:

| Aspect | assign | always_comb |
| --- | --- | --- |
| Syntax | one-line expression | procedural block |
| Drives | net (wire, logic) | variable (logic, reg) |
| Size | tiny glue logic | large case/if trees |
| Guarantees | evaluated continuously | tool adds sensitivity list automatically |
| Synthesis | identical when used correctly | identical |

always_comb shines when the logic needs multi-line if/else or case constructs; assign is ideal for concise expressions.

4 Best-practice checklist

- Keep modules small & single-purpose—they map directly to test-benches and block diagrams.

- Expose intent with parameters, e.g. bus width, to make reuse trivial.

- Prefer logic over wire until you truly need multiple drivers.

- Group related assigns together to ease reading; comment wide expressions.

- Simulate early; even simple combinational code can hide width mismatches that catch you only in synthesis.

5 Hands-on demo – 4-bit adder & comparator

Objective

Use nothing but continuous assignments to add two 4-bit numbers from the DE10-Lite switches, show the 5-bit result on LEDs, and light LED9 when the operands are equal.

Hardware map

- SW[3:0] – operand A

- SW[7:4] – operand B

- LEDR[4:0] – {carry, SUM[3:0]}

- LEDR[9] – equality flag

Step-by-step

module adder_eq_top

(input  logic [9:0]  SW,

output logic [9:0]  LEDR);

logic [4:0] sum;                   // 5-bit to hold carry

assign sum       = SW[3:0] + SW[7:4];              // adder

assign LEDR[4:0] = sum;                            // show result

assign LEDR[9]   = (SW[3:0] == SW[7:4]);           // comparator

assign LEDR[8:5] = '0;  // this format is auto width

endmodule

- Create a new Quartus project or copy into your hello-world build.

- Compile; there is no timing path longer than one adder so it meets ∼300 MHz easily on MAX 10.

- Program the .sof through the USB-Blaster.

- Flip switches and watch:

  - 0011 + 0100 lights LEDs 00111 (7) and equality LED stays dark.

  - Matching operands light only LED9.

This demo reinforces how a few assign lines synthesize straight into LUTs and carry-chains—perfect groundwork before you tackle always_ff state machines in the next lesson. (nios.montana.edu, UCD ECE)

# always_ff, always_latch & Sequential Logic Basics

FPGAs only become truly useful once you can store state inside them. SystemVerilog gives you two purpose-built procedural blocks for that job—always_ff for flip-flops and always_latch for level-sensitive latches—plus a handful of simple rules that keep simulation and synthesis in lock-step. This chapter explains how those blocks work, how they differ from plain always @(*), and the sequential-logic concepts (clocking, reset, metastability, non-blocking assignments) that every RTL designer must master. A short “one-shot pulse stretcher” demo shows the theory in action.

1 Why specialised blocks?

Traditional Verilog’s generic always @(posedge clk or …) can describe anything, so accidental race conditions or missed sensitivity items often slip past simulation and reappear as bugs in hardware (Stack Overflow).
SystemVerilog’s flavoured blocks (always_ff, always_latch, always_comb) impose stricter syntax that catches these errors at compile time

2 always_ff—sequential logic in one line

always_ff @(posedge clk or negedge rst_n) begin

if (!rst_n)  q <= '0;          // asynchronous active-low reset

else         q <= d;

end

- Use non-blocking <= so old and new values are both visible during the current time-step (Verification Academy, ChipVerify).

Asynchronous vs. synchronous reset

Placing rst in the event list yields an asynchronous reset that dominates the clock; putting the test inside the clocked branch yields a synchronous reset (markharvey.info).
Most FPGAs map either style cleanly, but synchronous resets ease timing closure because they obey the clock tree (Stack Overflow).

3 always_latch—only when you really need it

always_latch

if (en)  q <= d;      // level-sensitive when enable is high

- Triggers whenever any RHS signal changes; if an if/else chain leaves q unassigned, the compiler requires a latch (VLSIQUEST).

- Because latches are transparent during part of the clock cycle they complicate static timing; avoid them unless you are implementing asynchronous handshakes, retention registers or low-power gating (VLSIQUEST).

4 Sequential-logic fundamentals

4.1 Clocks and CE

- Distribute one clean, dedicated clock to every always_ff.

- Use a clock-enable (en) signal rather than gating the clock net; FPGA fabrics insert enable muxes into the flip-flop itself, saving power without skew penalties (Verilog Pro).

4.2 Blocking (=) vs. non-blocking (<=)

- In sequential blocks, always choose non-blocking to avoid accidental read-before-write behaviour; blocking assignments are fine inside always_comb for combinational logic

5 Common coding patterns

| Goal | Template | Notes |
| --- | --- | --- |
| Edge-detect (pulse on rising input) | logic in_d, in_q; always_ff @(posedge clk) begin in_q <= in_d; pulse <= in_d & ~in_q; end | Generates one-cycle “hit” |
| Load-enable register | always_ff @(posedge clk) if (load) q <= d; | Synth maps to mux-before-FF |
| Counter with async clear | always_ff @(posedge clk or posedge clr) if (clr) cnt <= '0; else cnt <= cnt + 1; | Avoid mixing async set and reset on the same FF (Reddit) |

6 Hands-on demo—pulse stretcher

Objective

Button presses on KEY[0] are often shorter than one clock; stretch each press into a fixed-length 10 ms pulse and show it on LEDR[0].

Coding hint

module pulse_stretcher

(input  logic        clk,         // 50 MHz

input  logic        rst_n,

input  logic        key_n,       // active-low push-button

output logic        pulse);      // 10 ms high

logic key_sync, key_last;

logic [18:0] cnt;                 // 10 ms @ 50 MHz ≈ 500 000 cycles

// 2-FF synchroniser

always_ff @(posedge clk or negedge rst_n) begin

if (!rst_n) {key_last,key_sync} <= 2'b11;

else        {key_last,key_sync} <= {key_sync, key_n};

end

// edge detect + monostable

always_ff @(posedge clk or negedge rst_n) begin

if (!rst_n)       {pulse,cnt} <= 0;

else if (!key_last && key_sync) begin     // falling edge

pulse <= 1;

cnt   <= 19'd499_999;

end

else if (cnt != 0) begin

cnt   <= cnt - 1;

pulse <= 1;

end

else

pulse <= 0;

end

endmodule

Shows synchronisation, non-blocking updates, a counter and clean reset—all staples of sequential RTL.

7 Checklist before synthesis

- One clock per domain; no inferred latches unless intentional.

- Every sequential signal assigned exactly once inside an always_ff.

- Use non-blocking assignments; simulate a test-bench that checks reset, enable, and rollover cases.

- Run the Timing Analyzer after fitting to verify FF-to-FF paths meet the 20 ns period (50 MHz).

Mastering always_ff, always_latch, and the core principles above turns messy “gate wiring” into clear, self-checking RTL—and prepares you for multi-clock CDC, pipelined datapaths, and every more-advanced design that follows.

# Blocking vs Non-blocking Assignments

Blocking (=) and non-blocking (<=) assignments look like a one-character difference, yet they follow completely different scheduling rules in simulation and lead to very different hardware when synthesized. In practice you use blocking for “instant” combinational logic and non-blocking for edge-triggered sequential logic; mixing them carelessly is a common source of race conditions and mismatches between simulation and the silicon. This chapter explains what each operator really does, shows when to choose one over the other, flags the classic pitfalls, and ends with a short lab that lets you see the effects directly on the DE10-Lite.

Conceptual overview

Blocking assignments (=)

- Execute immediately; the right-hand expression is evaluated and the left-hand variable is updated before the next statement in the same procedural block is reached.

- Because later code in the same block sees the new value, blocking assignments model combinational logic well inside always_comb or always @(*) blocks.

Non-blocking assignments (<=)

- Schedule an update for the end of the current simulation time-step; code that follows in the same block still sees the old value until the time-step advances.

- Intended for edge-triggered sequential logic inside always_ff @(posedge clk …) so that every register in the design samples inputs simultaneously.

How simulators really schedule the two operators

- Evaluate blocking statements in source order; each completes before moving on. (asic-world.com)

- Queue non-blocking targets in the NBA (non-blocking assignment) queue; updates occur after every block at the same time-step finishes, preventing accidental read-before-write races between two clocked processes. (sunburst-design.com)

- The IEEE 1800-2017 LRM defines this three-phase event flow explicitly (active, inactive, NBA). (rfsoc.mit.edu)

When to use which operator

| Use-case | Recommended operator | Why |
| --- | --- | --- |
| Pure combinational logic inside an always_comb | Blocking (=) | Mirrors continuous hardware evaluation; easier to read and avoids unintentional latches. (Nandland) |
| Clocked registers, counters, FSM state | Non-blocking (<=) | Ensures all registers load their next value on the same edge and avoids simulation races. (Xilinx Docs) |
| Temporary variables inside a clocked block before the first <= | Optional blocking | Often used as scratch signals, but never assign the same variable with both operators. (Stack Overflow) |
| Never mix in one block | — | Intel, Xilinx, and most foundry checkers flag mixed operators as an error. (Intel, Xilinx Docs) |

Classic pitfalls & race conditions

- Read-before-write race: two clocked always blocks write and read the same register with blocking assignments; simulation order determines the value, but hardware will wire OR the two drivers. (stitt-hub.com)

- Mixed assignment in one block: some tools allow it, but re-synthesis in another tool may reorder statements and change behaviour. (Intel, Xilinx Support)

- Latch inference: forgetting an else branch with blocking assignments in a supposedly combinational block causes the synthesizer to infer a level-sensitive latch. (asic-world.com)

Best-practice checklist

- One operator per procedural block—choose at the top and stay consistent.

- Combinational = blocking, sequential = non-blocking; your code becomes

- Never assign the same variable with both operators anywhere in the design; most linters flag this.

- Add reset logic only in always_ff; avoid asynchronous resets written with =.

Hands-on demo – see it fail and see it fixed

A race-prone version (for exploration)

// WARNING: bad style – mixes operators

always @(posedge clk) begin

a =  b;          // blocking

b <= a;          // non-blocking

end

Compile and simulate: depending on tool and optimise level, a and b might swap or latch the same value—hardware will not.

Corrected version

always_ff @(posedge clk) begin

a <= b;          // sequential

b <= a;          // sequential

end

Re-run the test-bench: both registers now update exactly as intended every clock tick.

Deploy the corrected RTL on the DE10-Lite, connect SW[0] to a, SW[1] to b, and watch LEDR[0]/LEDR[1] toggle cleanly with the clock—no meta-behaviour.

Further reading

- Cliff Cummings, “Non-blocking Assignments in Verilog Synthesis” – the definitive paper on NBA scheduling. (sunburst-design.com)

- Intel® Quartus® Design Recommendations 5.4.9 – why the Pro synthesis engine rejects mixed operators. (Intel)

- Xilinx UG901 Vivado Synthesis – concise mixing rules and simulation caveats. (Xilinx Docs)

- ASIC-World Tutorial on Blocking/Non-blocking – interactive examples and quizzes. (asic-world.com)

Follow these guidelines and you will avoid the majority of “worked in simulation, not on the board” bugs that plague first-time RTL developers.

# Case Statements & Priority Encoders

A case statement chooses exactly one branch out of many based on a value; combine it with the SystemVerilog keywords unique and priority and you can tell the synthesiser whether those branches must be parallel (no overlaps) or priority-encoded (first match wins). That controlled ordering is what turns a simple multiplexer into a hardware priority encoder—the building block that finds the first asserted bit in a vector. Mastering both constructs lets you write readable, timing-friendly RTL and avoid the “full/parallel-case evil twins” that once plagued classic Verilog.

1 Case-statement fundamentals

The plain case in SystemVerilog compares an expression against a list of case items and executes the first match it encounters. Because the list is scanned top-to-bottom, naïvely written code already implies priority: earlier items dominate later ones.
To make intent explicit—and allow tools to check it—SystemVerilog adds modifiers:

| Keyword | Guarantee | Typical hardware |
| --- | --- | --- |
| unique | At most one item may match; overlap is a compile-time error | Parallel multiplexer |
| unique0 | Same as unique but allows no match (implicit default) | Parallel mux with “no valid” output |
| priority | First matching item wins; later overlaps allowed | Priority encoder / arbiter |

Using these keywords eliminates the synthesis-directive hacks (/* parallel_case */, /* full_case */) that older code relied on sunburst-design.comsunburst-design.com.

2 Writing priority encoders with priority case

A priority encoder outputs the index of the highest-priority asserted input.
An 8-to-3 encoder, for example, maps in[7]…in[0] to a 3-bit code where bit 7 outranks bit 6 etc. GeeksforGeeksStack Overflow

module enc8to3_pri (input  logic [7:0] in,

output logic [2:0] code,

output logic       valid);

priority casez (in)           // ← explicit priority

8'b1???????: {valid,code} = 4'b1_111; // in[7]

8'b01??????: {valid,code} = 4'b1_110; // in[6]

8'b001?????: {valid,code} = 4'b1_101; // in[5]

8'b0001????: {valid,code} = 4'b1_100; // …

8'b00001???: {valid,code} = 4'b1_011;

8'b000001??: {valid,code} = 4'b1_010;

8'b0000001?: {valid,code} = 4'b1_001;

8'b00000001: {valid,code} = 4'b1_000;

default    : {valid,code} = 4'b0_000; // none set

endcase

endmodule

- priority tells both simulator and synthesiser to keep the written order

- casez treats ? as “don’t-care”, reducing typing and LUT count

- The valid flag avoids illegal codes when all inputs are zero.

3 Best-practice checklist

- Add default: to every case to stop latch inference and catch unplugged states

- Prefer unique/priority keywords to the deprecated full_case/parallel_case attributes

- Keep encoders ≤16 inputs for one-cycle speed; larger widths benefit from hierarchical encoders or synthesiser options that build logarithmic trees Intel CDRD.

4 Hands-on demo – 8-to-3 priority encoder on the DE10-Lite

A - Open DE10_lite_17_counter.qar, compile, burn and fix the bug..

B -

Goal

Use the board’s eight slide-switches as encoder inputs, show the 3-bit code on the red LEDs, and blank the HEX0 seven-segment display when no switch is on.

Pin map

| Board signal | Function |
| --- | --- |
| SW[7:0] | Encoder inputs (SW7 = highest priority) |
| LEDR[2:0] | Encoded output |
| LEDR[9] | valid flag |
| HEX0 | 7-segment decoder of code (off when valid = 0) |

Steps

- Create a new Quartus project, target device 10M50DAF484C7.

- Add enc8to3_pri.sv (above) and a small seven-segment decoder module.

- Compile; fitter should report <30 logic elements at 300 MHz.

- Program the .sof to the board.

- Move just one switch at a time: LED code counts from 000 (SW0) to 111 (SW7). With multiple switches up, only the highest one is recognised—demonstrating priority.

Optional: TimeQuest can reveal the single-cycle critical path (≈ 3 ns) and confirm that Quartus inserted a priority chain, not a large parallel mux Intel CDRD.

What you learned

- How priority case offers concise, self-checked RTL for encoders and arbiters.

- Why unique/priority keywords beat old synthesis pragmas for portability.

- How to map an 8-to-3 encoder onto real FPGA I/O and verify its behaviour in hardware.

Keep this encoder handy—you will need the same pattern later to build VGA sprite collision detectors and CPU interrupt arbiters.

# Finite State Machines: Moore & Mealy Designs

Finite-state machines (FSMs) let hardware “remember” context and react over time—vital for protocols, control loops, graphics, and games. SystemVerilog makes them readable and synthesize-friendly with enumerated types, split state/next-state/output logic, and the specialised always_ff / always_comb blocks you met in earlier chapters. Two classic flavours dominate: Moore machines, whose outputs depend only on the current state, and Mealy machines, whose outputs can also depend on current inputs. Each has timing and resource trade-offs you should understand before tackling larger projects. (GeeksforGeeks, Intel)

Moore vs Mealy—conceptual recap

- Moore FSM: outputs are registered alongside the state, so they change one clock after a state transition; this simplifies timing analysis and avoids combinational glitches. (GeeksforGeeks)

- Mealy FSM: outputs can toggle in the same cycle that an input changes because they’re a function of state & inputs; this yields faster reaction with fewer states but demands careful glitch handling. (VLSI Verify, Reddit)

FPGA syntheses default to one-hot encoding (one flip-flop per state) because it maps well to LUT architectures; other styles—binary, gray, Johnson—remain selectable for power or area tuning. (Intel, Intel Community)

Coding pattern (recommended by Intel & Xilinx)

typedef enum logic [2:0] {S_RED, S_RG, S_GREEN} state_t;

state_t  state, next;

///////////////////////////////////////

// 1) state register

always_ff @(posedge clk or negedge rst_n)

if (!rst_n)  state <= S_RED;

else         state <= next;

///////////////////////////////////////

// 2) next-state & Mealy outputs

always_comb begin

next      = state;       // default

walk_req  = 0;           // Mealy output

unique case (state)

S_RED  : if (timer_exp) next = S_RG;

S_RG   : if (timer_exp) next = S_GREEN;

S_GREEN: begin

if (btn)    walk_req = 1;

if (timer_exp)       next = S_RED;

end

endcase

end

///////////////////////////////////////// 3) Moore outputs

assign {red, amber, green} =

(state==S_RED  ) ? 3'b100 :

(state==S_RG   ) ? 3'b110 :

3'b001 ;

This “three-block” template is Intel’s* strongly recommended style for safe FSM synthesis. (Intel, Xilinx Support)*

Example 1 — Moore traffic-light controller

A traffic intersection needs rock-solid outputs that never glitch; a Moore machine is ideal. The code above drives three LEDs (red/amber/green) solely from the registered state, while the pedestrian request (walk_req) is a Mealy-style signal proving you can mix both philosophies in one design. Synthesise in Quartus and open Tools ▸ Netlist Viewers ▸ State Machine Viewer to verify one-hot encoding and state diagram. (YouTube, Intel)

Example 2 — Mealy 1011 sequence detector

Detecting a four-bit pattern in a serial stream benefits from Mealy logic—fewer states and one-cycle earlier assertion:

module seq1011

(input  logic clk, rst_n, din,

output logic hit);

typedef enum logic [2:0] {S0, S1, S10, S101} s_t;

s_t state,next;

always_ff @(posedge clk, negedge rst_n)

if (!rst_n) state <= S0;

else        state <= next;

always_comb begin

unique case (state)

S0  : {hit,next} = din ? {0,S1}   : {0,S0};

S1  : {hit,next} = din ? {0,S1}   : {0,S10};

S10 : {hit,next} = din ? {0,S101} : {0,S0};

S101: {hit,next} = din ? {1,S1}   : {0,S10}; // Mealy output

endcase

end

endmodule

Outputs (hit) are generated inside the always_comb block, giving an immediate match when the last “1” arrives. Quartus synthesises this as four FFs plus LUT logic, identical to textbook diagrams. (VLSI Verify)

Implementation tips

| Checklist item | Rationale |
| --- | --- |
| Use unique or priority on case to let tools flag overlap/coverage bugs. (sunburst-design.com, markharvey.info) |  |
| Register every external input once before feeding an FSM to kill metastability. (Goddard Engineering Directorate) |  |
| Add a default branch that reverts to a safe state; reviewers & formal tools love it. (vhdl-online.de) |  |
| Confirm encoding in the Quartus State-Machine Viewer; switch to binary if you’re tight on FFs. (Intel, Intel) |  |

Challenge — “Reaction-Timer” Game (DE10-Lite Capstone)

Goal: Design a Moore-style FSM that lights HEX0 countdown “3-2-1-GO”. When GO appears, the user must press KEY0 within 250 ms. Show “WIN” on the 7-segment if successful, “LO” if too early/late.

Constraints

- Use exactly two always blocks (always_ff + always_comb).

- Drive the 7-segment with registered outputs only (pure Moore).

- Provide a synchronised, debounced version of KEY0.

- Bonus: display reaction time in milliseconds on HEX4..1.

Implement, simulate, and then time the path with SignalTap to verify no combinational feedback exists. Share a screenshot of your state diagram from the Quartus State-Machine Viewer and your maximum fMAX.

What you’ll practise: enumerated states, one-hot vs binary encoding, timer counters, and glitch-free output logic—everything a real project demands.

Master your Moore and Mealy patterns now; they form the heartbeat of every VGA controller, UART, or game engine you’ll build in the rest of this course.